LAB EXPERIMENTS

ECA1410 – EMBEDDED SYSTEM

EXP – 1

FLASHING OF LED USING AT89C51 MICROCONTROLLER USING PROTEUS

AIM:

To Write an assembly language program to flash the LED using AT89C51

SOFTWARES REQUIRED:

• Proteus software

PROGRAM

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

EXP- 2:

GENERATION OF SQUARE WAVE USING PROTEUS

AIM:

To write an assembly language program to Generate square wave using

AT89C51.

SOFTWARE REQUIRED:

• Proteus 8 software

PROGRAM

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

EXP – 3:

AIM:

To write an assembly language program for Stepper Motor Using AT89C51 using Keil and

Proteus

SOFTWARE REQUIRED:

• Proteus 8 software.

PROGRAM:

ORG 0000H

UP: MOV P2,#09H

ACALL DELAY

MOV P2,#0CH

ACALL DELAY

MOV P2,#06H

ACALL DELAY

MOV P2,#03H

ACALL DELAY

SJMP UP

DELAY:MOV R4,#18

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

Exp No: 4

7 SEGMENT DISPLAY USING AT89C51 USING PROTEUS

AIM:

To write an assembly language program for 7 Segment Display Using AT89C51 using Keil and Proteus

SOFTWARE REQUIRED:

• Proteus 8 software.

PROGRAM:

ORG 000H

UP:MOV P2,#0C0H

ACALL DELAY

MOV P2,#0F9H

ACALL DELAY

MOV P2,#0A4H

ACALL DELAY

MOV P2,#0B0H

ACALL DELAY

MOV P2,#99H

ACALL DELAY

MOV P2,#92H

ACALL DELAY

MOV P2,#82H

ACALL DELAY

MOV P2,#0F8H

ACALL DELAY

MOV P2, #80H

ACALL DELAY

MOV P2,#90H

ACALL DELAY

DELAY: MOV R5,#10

H1:MOV R4,#180

H2:MOV R3,#255

H3:DJNZ R3,H3

DJNZ R4,H2

DJNZ R5,H1

RET

END

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

Exp No: 9

ARDUINO BASED MANUAL ELECTRONIC COUNTER USING

PROTEUS AIM:

To write an Embedded C program for Manual Electronic counter using Arduino IDE and

Proteus

SOFTWARE REQUIRED:

• Proteus 8 software

• Arduino IDE

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

Exp No: 6

ARDUINO TO 16x2 LCD DISPLAY USING PROTEUS

AIM:

To write an Embedded C program to interface Arduino Uno and 16 x 2 LCD display using

Arduino Uno and Proteus

SOFTWARE REQUIRED:

• Proteus 8 software.

• Arduino IDE software

PROGRAM:

Arduino to 16x2 LCD Display using Proteus

#include &lt;LiquidCrystal.h&gt;

LiquidCrystal lcd (7, 8, 9, 10, 11, 12);

// your PWM numbers may be different

void setup() {

lcd.begin(16, 2);

lcd.print(&quot;put your message here&quot;);

}

void loop () {

for (int i=0; 1&lt;13; i++)

lcd.scrollDisplayLeft();

delay(700);

}

}

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

Exp No: 8

GAS SENSOR MQ-2 IN PROTEUS USING ARDUINO

AIM:

To write an Embedded C program for interfacing Gas Sensor MQ-2 using Arduino Uno and Proteus

SOFTWARE REQUIRED:

• Proteus 8 software.

• Arduino IDE software

PROGRAM:

#define GreenLed 13

#define Sensor A0

void setup(){

pinMode(13,OUTPUT);

pinMode(12,OUTPUT);

pinMode(A0,INPUT);

Serial.begin(9600);

}

void loop() {

int value = analogRead(A0);

Serial.print(&quot;Analogic Value coming from the sensor : &quot;);

Serial.println(value);

delay(100);

if(value&gt;600){

digitalWrite(13,HIGH);

}

else{

digitalWrite(13,LOW);

}

delay(20);

}

Code for MQ6 sensor:

int mq6Pin = A0; // MQ-5 analog pin connected to A0

void setup() {

Serial.begin(9600);

void loop() {

int sensorValue = analogRead(mq5Pin);

Serial.print(&quot;MQ5 Sensor Value: &quot;);

Serial.println(sensorValue);

delay(1000); // 1-second delay

}

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

Exp No: 7

SERIAL COMMUNICATION USING ARDUINO WITH

PROTEUS AIM:

To write an Embedded C program for serial communication using Arduino Uno and Proteus

SOFTWARE REQUIRED:

• Proteus 8 software.

• Arduino IDE software

PROGRAM:

void setup()

{

Serial.begin(9600); // send and receive at 9600 baud

}

int number = 0;

void loop()

{

Serial.print(&quot;Number is &quot;);

Serial.println(number); // print the number

delay(500); // delay half second between numbers

number++; // to the next number

}

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

Exp No:

ULTRASONIC SENSOR IN PROTEUS USING ARDUINO

AIM:

To write an Embedded C program for interfacing Ultrasonic Sensor using Arduino Uno

and Proteus

SOFTWARE REQUIRED:

• Proteus 8 software.

• Arduino IDE software

PROGRAM:

// Program Sketch

int trig = 10;

int echo = 9;

long duration;

int cm;

void setup() {

// put your setup code here, to run once:

pinMode(trig, OUTPUT);

pinMode(echo,INPUT);

Serial.begin(9600);

}

void loop() { // Sketch to calculate ultrasonic distance

digitalWrite(trig, LOW);

delayMicroseconds(10);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

delayMicroseconds(10);

duration = pulseIn(echo, HIGH);

cm = (duration/2) * 0.034;

// Print Value on Serial Monitor

Serial.print(&quot; Distance = &quot;);

Serial.print(cm);

Serial.println(&quot; cm&quot;);

}

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.

Exp No: 10

PIR SENSOR IN PROTEUS USING ARDUINO

AIM:

To write an Embedded C program for interfacing PIR Sensor using Arduino Uno and Proteus

• Proteus 8 software.

• Arduino IDE software

PROGRAM:

int Status = 13; // Digital pin D6

int sensor = 12; // Digital pin D7

void setup() {

Serial.begin(9600);

pinMode(sensor, INPUT); // declare sensor as input

pinMode(Status, OUTPUT); // declare LED as output

}

void loop() {

long state = digitalRead(sensor);

Serial.println(state);

if(state == HIGH) {

digitalWrite (Status, HIGH);

Serial.println(&quot;Motion detected!&quot;);

delay(1000);

}

else {

digitalWrite (Status, LOW);

Serial.println(&quot;Motion absent!&quot;);

delay(1000);

}

}

OUTPUT:

RESULT:

Thus the program has been successfully verified and executed.